Green Space Double-Cab Truck Specifications

Blacktown City Council

City Fleet

City Assets

7.5T GVM, Double cab, AMT, Tipper, Toolbox

- Configuration: Double-cab chassis

- GVM (Gross Vehicle Mass): 7.5 T

- Truck cab Colour: OEM White

- Transmission: AMT (Automated Manual Transmission) or equivalent

- Wheelbase: 3850mm or similar

- Emission level: must meet or exceed ADR 80/04 (Euro 6)

- Features:

  - Air Conditioning

  - Electric heated mirrors

  - Bluetooth hands-free system

  - Two sets of keys

  - Extra seat padding for front and rear fixed seats

  - Reversing camera and reversing alarm

  - Tinted windows to NHVR limits

  - Yellow grab handles in cab

  - Yellow steps on cabin exterior

  - Hydraulic cabin lift mechanism

Tipper Body Specifications:

- Capacity: 3 Tonnes

- Construction: High-Density (HD) steel construction

- Dimensions: Approximately 2400 mm (L) x 2200 mm (W), unless otherwise specified, the overall width of the body shall be no more than 200mm wider than the truck cab

- Features:

  - 450mm high drop sides with fixed corner posts and a 600mm two-way tailgate

  - Lift weight on drop sides not to exceed 20 Kg

  - 5mm smooth steel floor

  - 40mm tailgate lock bar with 4 locking lugs and mechanical lock(s)

  - Replaceable Heavy-Duty tip body hinges

  - Multi-stage under-body hoist with safety prop painted safety-yellow (H-Frame)

  - Tie rails either side below tray level

  - Off-road pin type tow bar (Cruisemaster D035 or equivalent) with 12V 7-pin small round coupling socket and, if the trailer electric brakes are fitted, 12-pin flat connector (connected in accordance with Australian Standard AS4177.5-2004)

  - Tow bar fully integrated under the bodyline to prevent damage to the tow bar and to avoid obstruction during the tipping process.

  - Electric trailer brake module with in-cab battery level indicator, a breakaway monitoring system, and charging system with a charging indicator in cabin if required.

  - Battery isolator switch

  - If the trailer equipped with electric brakes, an Anderson plug shall be installed at rear end for charging trailer’s battery.

Additional Features:

- Foldable, non-slip climbing side-step (non-slip, minimum dimensions of 300 mm depth and 400mm width) inside passenger’s side (LHS) front drop gate with a handrail on the front board, all painted safety-yellow for visibility and safety

Rear-Facing Arrow Sign:

- A rear-facing, Type B, single-sided electric folding flashing arrow sign mounted above the load board.

- Equip the sign with two (Class 1 rated) rotating amber beacon lights, one on each side, beacon lights should be installed at a height to ensure visibility for motorists immediately behind the truck, enhancing overall visibility

Toolbox and Access Steps:

- Fixed toolbox dimensions: 2200mm (L) x 1400mm (H) x 900mm (W), located between the cabin and load board, mounted on the chassis rail

- Toolbox features:

  - Toolbox shall be fabricated with 3mm aluminium plate

  - Swing open, lockable, waterproof barn doors with metal lip (instead of rubber seal), and gas struts

  - One shelf positioned 600mm below the top of the box. The top half of the toolbox should be divided in the middle by a vertical aluminium plate.

  - All shelves constructed from 3mm thick aluminium, with rolled edges (10mm lip)

  - A fold out step below the toolbox on both sides, and handrails inside the toolbox all painted safety-yellow, to access the top shelf with minimum 600mm clearance

  - Whale-tail type locks on all doors

- Toolbox Ventilation:

- Air vents on the doors of the toolbox, ensuring optimal airflow and ventilation within the toolbox

Water Tank:

- Incorporate a water tank with a 50-liter capacity, equipped with a breather filler cap and a dispensing tap.

- The tank should be mounted at a suitable position on the passenger side of the vehicle. The tank shall be located a minimum of 600mm above ground level. Water tank should be labelled “NOT FOR DRINKING”.

- A soap dispenser should be mounted near the water dispensing tap for convenient access.

Tipper Body Paint and Coating:

- The exterior of the tipper body should be painted in "OEM Cab Chassis white" using 2PAK colour paint.

- Leave the toolboxes unpainted to maintain their original material finish.

- The interior of the tipper body should be coated with a black rustproof epoxy to prevent corrosion.

Additional Features:

- Rubber flap mudguards to protect against road debris and splashes

Paint and Coating:

- All access steps, grab handles, handrails, and safety props provided shall be painted safety yellow-Paint code Y15.

Signage:

- All lettering used to indicate the vehicle’s GVM, GCM and Tare mass shall be at least 50mm in size ad be of Commercial Grade.

- All trucks must comply to VSB12 (Rear Marker Plate)

Custom paint:

- Custom colours to the bodies and toolboxes (colours to be advised) (Price as option)

- Brand guidelines and the colour codes of our logo should be used